M A T E R I A L  S A F E T Y  D A T A  S H E E T

SECTION 1. - - - - - - - - - CHEMICAL IDENTIFICATION- - - - - - - - - -

NAME:                 GERMANIUM(IV) CHLORIDE, 99.99999%

SECTION 2. - - - - - COMPOSITION/INFORMATION ON INGREDIENTS - - - - - -

CAS #: 10038-98-9

MF: CL4GE

EC NO:  233-116-7

SYNONYMS

GERMANIUM TETRACHLORIDE * GERMANIUM, TETRACHLORO- *

SECTION 3. - - - - - - - - - - HAZARDS IDENTIFICATION - - - - - - - - -

LABEL PRECAUTIONARY STATEMENTS

CORROSIVE

CAUSES BURNS.

IRRITATING TO EYES AND RESPIRATORY SYSTEM.

REACTS VIOLENTLY WITH WATER.

POSSIBLE RISK OF IRREVERSIBLE EFFECTS.

POSSIBLE MUTAGEN.

LACHRYMATOR.

IN CASE OF CONTACT WITH EYES, RINSE IMMEDIATELY WITH PLENTY OF

WATER AND SEEK MEDICAL ADVICE.

AFTER CONTACT WITH SKIN, WASH IMMEDIATELY WITH PLENTY OF WATER.

TAKE OFF IMMEDIATELY ALL CONTAMINATED CLOTHING.

WEAR SUITABLE PROTECTIVE CLOTHING, GLOVES AND EYE/FACE

PROTECTION.

STORE UNDER NITROGEN.

SECTION 4. - - - - - - - - - - FIRST-AID MEASURES- - - - - - - - - - -

IN CASE OF CONTACT, IMMEDIATELY FLUSH EYES OR SKIN WITH COPIOUS

AMOUNTS OF WATER FOR AT LEAST 15 MINUTES WHILE REMOVING CONTAMINATED

CLOTHING AND SHOES.

ASSURE ADEQUATE FLUSHING OF THE EYES BY SEPARATING THE EYELIDS

WITH FINGERS.

IF INHALED, REMOVE TO FRESH AIR. IF NOT BREATHING GIVE ARTIFICIAL

RESPIRATION. IF BREATHING IS DIFFICULT, GIVE OXYGEN.

IF SWALLOWED, WASH OUT MOUTH WITH WATER PROVIDED PERSON IS CONSCIOUS.

CALL A PHYSICIAN.

WASH CONTAMINATED CLOTHING BEFORE REUSE.

DISCARD CONTAMINATED SHOES.

SECTION 5. - - - - - - - - - FIRE FIGHTING MEASURES - - - - - - - - - -

EXTINGUISHING MEDIA

CARBON DIOXIDE, DRY CHEMICAL POWDER OR APPROPRIATE FOAM.

DO NOT USE WATER.

SPECIAL FIREFIGHTING PROCEDURES

WEAR SELF-CONTAINED BREATHING APPARATUS AND PROTECTIVE CLOTHING TO

PREVENT CONTACT WITH SKIN AND EYES.

UNUSUAL FIRE AND EXPLOSIONS HAZARDS

EMITS TOXIC FUMES UNDER FIRE CONDITIONS.

SECTION 6. - - - - - - - - ACCIDENTAL RELEASE MEASURES- - - - - - - - -

EVACUATE AREA.

WEAR SELF-CONTAINED BREATHING APPARATUS, RUBBER BOOTS AND HEAVY

RUBBER GLOVES.

ABSORB ON SAND OR VERMICULITE AND PLACE IN CLOSED CONTAINERS FOR

DISPOSAL.

VENTILATE AREA AND WASH SPILL SITE AFTER MATERIAL PICKUP IS COMPLETE.